Nuclear extraction Last modified 11.01.2012

Materials

- Phosphate buffered saline (PBS) - Buffer A (low salt): 10mM HEPES-KOH pH 8.0, 1.5 mM MgCl2, 10 mM NaCl - Buffer A*: Buffer A + protease inhibitors + 0.15% NP40 - Buffer C (high salt): 20mM HEPES-KOH pH 8.0, 2 mM MgCl2, 420 mM NaCl, 20% (v/v) glycerol, 0.2 mM EDTA, 0.1% (v/v) NP40. Add fresh before use: 0.5 mM DTT, complete protease inhibitors mini EDTA free (1 tablet/10 ml) (Roche 11836170001) - Fresh cells (not frozen pellets)

Equipment

- Glass douncer with type B pestle (in 500 µl, 2ml, 7 ml and 15 ml volumes) - Cooled bench centrifuge (Heraeus Multifuge 3 S-R; for 15, 5 or 500 ml tubes) - Cooled tabletop centrifuge (Heraeus Fresco 21; for 1.5 or 2 ml vials)

| Procedure | Comments |
| --- | --- |
| A) Harvesting cells (adherent cells) |  |
| - Wash cells with 10 ml PBS - Trypsinise with 1.5 ml trypsin/15 cm dish - Resuspend with 10 plates with 10 ml of medium - Pool in a 50 ml tube - Repeats steps 3-4 - Perform all subsequent steps at 4 °C/on ice - Centrifuge cells 5 min, 400 g, 4 °C. Remove supernatant | - Before starting, pre-cool buffer A, buffer A*, buffer C , PBS and the douncer on ice - For recommended douncer volumes, refer to step 24 comments - These steps may vary; in general, wash, trypsinise, neutralise, resuspend and pool |
| - Resuspend cells with 50 ml PBS - Centrifuge 5 min, 400 g, 4 °C. Remove supernatant - Repeat steps 8-9 - Resuspend cells with 8 ml PBS and transfer to the 15 ml tube - Rinse the 50 ml tube again with 5 ml PBS and transfer to the 15 ml tube - Centrifuge 5 min, 400 g, 4 °C. Remove supernatant |  |
| B) Harvesting cells (suspension cells) |  |
| - Centrifuge 500ml of cells in a 500ml tube for 5 min, 400 g, 4 °C. Remove supernatant - Resuspend cells with 50 ml PBS - Centrifuge 5 min, 400 g, 4 °C. Remove supernatant - Repeat steps 14-15 - Resuspend cells with 8 ml PBS and transfer to the 15 ml tube - Rinse the 50 ml tube again with 5 ml PBS and transfer to the 15 ml tube - Centrifuge 5 min, 400 g, 4 °C. Remove supernatant |  |
| Mechanical lysis of cells |  |
| - Determine cell pellet volume and resuspend cells with 5 volumes of buffer A - Incubate on ice for 15 min - Centrifuge 5 min, 400 g, 4 °C. Remove supernatant | - Cell pellet should swell after incubation with buffer A. Increase in cell volume may vary depending on cell type. - While waiting, wash douncer with demi water and rinse with buffer A. |

| - Determine cell pellet volume and resuspend in 2 volumes of buffer A* - Apply 3 x 10 to 4 x 10 strokes. Wait 30s after every set of strokes - Transfer the cell suspension to a 15 ml tube - An aliquot of cell suspension can be checked under a microscope to see whether the cells have lysed from douncing. Perform during step 28. | - Recommended douncers: - 0.1 ml douncer: >50 µl swelled pellet - 2 ml douncer: 50-600 µl swelled pellet - 7 ml douncer: 600-2500 µl swelled pellet - The plunger must remain in the cell suspension to avoid air bubbles - The 30s rests between sets of strokes prevent heating up of the cell suspension that may result in protein instability - Douncing opens up cells (but not nuclei) |
| --- | --- |
| - Centrifuge 15 min, 3200 g, 4 °C - Transfer or discard the supernatant (cytoplasmic extract (CE)) | - To store the CE, add glycerol (10% final conc.) and NaCl (150 mM final conc.). If the nuclear extraction has priority perform after step 29 |
| Lysis of nuclei |  |
| - Determine the volume of cell pellet (crude nuclei) - Wash the pellet with 10 volumes of PBS. The nuclei are very fragile. Quickly add the PBS and invert. Do not pipette up and down. - Centrifuge 5 min, 3200 g, 4 °C. Discard supernatant |  |
| - Determine the volume of cell pellet - Add 2 volumes of buffer C - Cut the tip of a 1 ml pipette and resuspend as best as possible by pipetting up and down vigorously - Transfer the suspension to 1.5 or 2 ml vials - Incubate 1 h, 4 °C on a rotating wheel | - After incubation, the suspension will become viscous and white clouds of chromatin should appear |
| - Centrifuge on a table-top centrifuge for 45 min, 20,800 g or max speed, 4 °C - Aliquot the supernatant (nuclear extract (NE)) and snap freeze with liquid nitrogen - Store at -80 °C | - An aliquot of NE can be analysed for protein conc. via Nanodrop. Expected conc.: ~5 mg/ml - The pellet contains chromatin |